/*

Rui Santos

Complete project details at https://RandomNerdTutorials.com/esp-mesh-esp32-esp8266-painlessmesh/

Permission is hereby granted, free of charge, to any person obtaining a copy

of this software and associated documentation files.

The above copyright notice and this permission notice shall be included in all

copies or substantial portions of the Software.

*/

#include <Adafruit_Sensor.h>

#include <Adafruit_BME280.h>

#include "painlessMesh.h"

#include <Arduino_JSON.h>

// MESH Details

#define   MESH_PREFIX     "AEMESH" //name for your MESH

#define   MESH_PASSWORD   "MESHpassword" //password for your MESH

#define   MESH_PORT       5555 //default port

//BME object on the default I2C pins

Adafruit_BME280 bme;

//Number for this node

int nodeNumber = 2;

//String to send to other nodes with sensor readings

String readings;

Scheduler userScheduler; // to control your personal task

painlessMesh  mesh;

// User stub

void sendMessage() ; // Prototype so PlatformIO doesn't complain

String getReadings(); // Prototype for sending sensor readings

//Create tasks: to send messages and get readings;

Task taskSendMessage(TASK_SECOND * 5 , TASK_FOREVER, &sendMessage);

String getReadings () {

JSONVar jsonReadings;

jsonReadings["node"] = nodeNumber;

jsonReadings["temp"] = bme.readTemperature();

jsonReadings["hum"] = bme.readHumidity();

jsonReadings["pres"] = bme.readPressure()/100.0F;

readings = JSON.stringify(jsonReadings);

return readings;

}

void sendMessage () {

String msg = getReadings();

mesh.sendBroadcast(msg);

}

//Init BME280

void initBME(){

if (!bme.begin(0x77)) {

Serial.println("Could not find a valid BME280 sensor, check wiring!");

while (1);

}

}

// Needed for painless library

void receivedCallback( uint32_t from, String &msg ) {

Serial.printf("Received from %u msg=%s\n", from, msg.c_str());

JSONVar myObject = JSON.parse(msg.c_str());

int node = myObject["node"];

double temp = myObject["temp"];

double hum = myObject["hum"];

double pres = myObject["pres"];

Serial.print("Node: ");

Serial.println(node);

Serial.print("Temperature: ");

Serial.print(temp);

Serial.println(" C");

Serial.print("Humidity: ");

Serial.print(hum);

Serial.println(" %");

Serial.print("Pressure: ");

Serial.print(pres);

Serial.println(" hpa");

}

void newConnectionCallback(uint32_t nodeId) {

Serial.printf("New Connection, nodeId = %u\n", nodeId);

}

void changedConnectionCallback() {

Serial.printf("Changed connections\n");

}

void nodeTimeAdjustedCallback(int32_t offset) {

Serial.printf("Adjusted time %u. Offset = %d\n", mesh.getNodeTime(),offset);

}

void setup() {

Serial.begin(115200);

initBME();

//mesh.setDebugMsgTypes( ERROR | MESH_STATUS | CONNECTION | SYNC | COMMUNICATION | GENERAL | MSG_TYPES | REMOTE ); // all types on

mesh.setDebugMsgTypes( ERROR | STARTUP );  // set before init() so that you can see startup messages

mesh.init( MESH_PREFIX, MESH_PASSWORD, &userScheduler, MESH_PORT );

mesh.onReceive(&receivedCallback);

mesh.onNewConnection(&newConnectionCallback);

mesh.onChangedConnections(&changedConnectionCallback);

mesh.onNodeTimeAdjusted(&nodeTimeAdjustedCallback);

userScheduler.addTask(taskSendMessage);

taskSendMessage.enable();

}

void loop() {

// it will run the user scheduler as well

mesh.update();

}