/*

Rui Santos

Complete project details at https://RandomNerdTutorials.com/esp32-mqtt-publish-bme280-arduino/

Permission is hereby granted, free of charge, to any person obtaining a copy

of this software and associated documentation files.

The above copyright notice and this permission notice shall be included in all

copies or substantial portions of the Software.

*/

#include <Wire.h>

#include <Adafruit_Sensor.h>

#include <Adafruit_BME280.h>

#include <WiFi.h>

extern "C" {

#include "freertos/FreeRTOS.h"

#include "freertos/timers.h"

}

#include <AsyncMqttClient.h>

#define WIFI_SSID "ThePatelHousehold"

#define WIFI_PASSWORD "jankitiki"

// Raspberry Pi Mosquitto MQTT Broker

#define MQTT_HOST IPAddress(127, 0, 1, 1)

// For a cloud MQTT broker, type the domain name

#define MQTT_HOST "STRNADBOUY"

#define MQTT_PORT 1883

// Temperature MQTT Topics

#define MQTT_PUB_TEMP "esp32/bme280/temperature"

#define MQTT_PUB_HUM "esp32/bme280/humidity"

#define MQTT_PUB_PRES "esp32/bme280/pressure"

// BME280 I2C

Adafruit_BME280 bme;

// Variables to hold sensor readings

float temp;

float hum;

float pres;

AsyncMqttClient mqttClient;

TimerHandle_t mqttReconnectTimer;

TimerHandle_t wifiReconnectTimer;

unsigned long previousMillis = 0;   // Stores last time temperature was published

const long interval = 10000;        // Interval at which to publish sensor readings

void connectToWifi() {

Serial.println("Connecting to Wi-Fi...");

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

}

void connectToMqtt() {

Serial.println("Connecting to MQTT...");

mqttClient.connect();

}

void WiFiEvent(WiFiEvent_t event) {

Serial.printf("[WiFi-event] event: %d\n", event);

switch(event) {

case SYSTEM_EVENT_STA_GOT_IP:

Serial.println("WiFi connected");

Serial.println("IP address: ");

Serial.println(WiFi.localIP());

connectToMqtt();

break;

case SYSTEM_EVENT_STA_DISCONNECTED:

Serial.println("WiFi lost connection");

xTimerStop(mqttReconnectTimer, 0); // ensure we don't reconnect to MQTT while reconnecting to Wi-Fi

xTimerStart(wifiReconnectTimer, 0);

break;

}

}

void onMqttConnect(bool sessionPresent) {

Serial.println("Connected to MQTT.");

Serial.print("Session present: ");

Serial.println(sessionPresent);

}

void onMqttDisconnect(AsyncMqttClientDisconnectReason reason) {

Serial.println("Disconnected from MQTT.");

if (WiFi.isConnected()) {

xTimerStart(mqttReconnectTimer, 0);

}

}

/*void onMqttSubscribe(uint16_t packetId, uint8_t qos) {

Serial.println("Subscribe acknowledged.");

Serial.print("  packetId: ");

Serial.println(packetId);

Serial.print("  qos: ");

Serial.println(qos);

}

void onMqttUnsubscribe(uint16_t packetId) {

Serial.println("Unsubscribe acknowledged.");

Serial.print("  packetId: ");

Serial.println(packetId);

}*/

void onMqttPublish(uint16_t packetId) {

Serial.print("Publish acknowledged.");

Serial.print("  packetId: ");

Serial.println(packetId);

}

void setup() {

Serial.begin(115200);

Serial.println();

// Initialize BME280 sensor

if (!bme.begin(0x77)) {

Serial.println("Could not find a valid BME280 sensor, check wiring!");

while (1);

}

mqttReconnectTimer = xTimerCreate("mqttTimer", pdMS_TO_TICKS(2000), pdFALSE, (void*)0, reinterpret_cast<TimerCallbackFunction_t>(connectToMqtt));

wifiReconnectTimer = xTimerCreate("wifiTimer", pdMS_TO_TICKS(2000), pdFALSE, (void*)0, reinterpret_cast<TimerCallbackFunction_t>(connectToWifi));

WiFi.onEvent(WiFiEvent);

mqttClient.onConnect(onMqttConnect);

mqttClient.onDisconnect(onMqttDisconnect);

//mqttClient.onSubscribe(onMqttSubscribe);

//mqttClient.onUnsubscribe(onMqttUnsubscribe);

mqttClient.onPublish(onMqttPublish);

mqttClient.setServer(MQTT_HOST, MQTT_PORT);

// If your broker requires authentication (username and password), set them below

//mqttClient.setCredentials("REPlACE_WITH_YOUR_USER", "REPLACE_WITH_YOUR_PASSWORD");

connectToWifi();

}

void loop() {

unsigned long currentMillis = millis();

// Every X number of seconds (interval = 10 seconds)

// it publishes a new MQTT message

if (currentMillis - previousMillis >= interval) {

// Save the last time a new reading was published

previousMillis = currentMillis;

// New BME280 sensor readings

temp = bme.readTemperature();

//temp = 1.8*bme.readTemperature() + 32;

hum = bme.readHumidity();

pres = bme.readPressure()/100.0F;

// Publish an MQTT message on topic esp32/BME2800/temperature

uint16_t packetIdPub1 = mqttClient.publish(MQTT_PUB_TEMP, 1, true, String(temp).c_str());

Serial.printf("Publishing on topic %s at QoS 1, packetId: %i", MQTT_PUB_TEMP, packetIdPub1);

Serial.printf("Message: %.2f \n", temp);

// Publish an MQTT message on topic esp32/BME2800/humidity

uint16_t packetIdPub2 = mqttClient.publish(MQTT_PUB_HUM, 1, true, String(hum).c_str());

Serial.printf("Publishing on topic %s at QoS 1, packetId %i: ", MQTT_PUB_HUM, packetIdPub2);

Serial.printf("Message: %.2f \n", hum);

// Publish an MQTT message on topic esp32/BME2800/pressure

uint16_t packetIdPub3 = mqttClient.publish(MQTT_PUB_PRES, 1, true, String(pres).c_str());

Serial.printf("Publishing on topic %s at QoS 1, packetId: %i", MQTT_PUB_PRES, packetIdPub3);

Serial.printf("Message: %.3f \n", pres);

}

}