#include "painlessMesh.h"

#include <Adafruit_Sensor.h>

#include <Adafruit_BME280.h>

#include <Arduino_JSON.h>

//include <WiFi.h>

#include "ThingSpeak.h"

#define   MESH_PREFIX     "AEMESH"

#define   MESH_PASSWORD   "MESHpassword"

#define   MESH_PORT       5555

const char* ssid = "US_Guest";   // your network SSID (name)

const char* password = "";   // your network password

WiFiClient  client;

unsigned long myChannelNumber = 1;

const char * myWriteAPIKey = "L7UT4TB0FA5CK7NG";

// Timer variables

unsigned long lastTime = 0;

unsigned long timerDelay = 10000;

// Variable to hold temperature readings

float temperatureC;

float humidity;

float pressure;

//uncomment if you want to get temperature in Fahrenheit

//float temperatureF;

//Number for this node

int nodeNumber = 18;

Scheduler userScheduler; // to control your personal task

painlessMesh  mesh;

//BME object on the default I2C pins

Adafruit_BME280 bme;

// Get a new temperature reading

void loop() {

if ((millis() - lastTime) > timerDelay) {

// Get a new temperature reading

temperatureC = bme.readTemperature();

Serial.print("Temperature (ºC): ");

Serial.println(temperatureC);

humidity = bme.readHumidity();

Serial.print("Humidity (%): ");

Serial.println(humidity);

pressure = bme.readPressure() / 100.0F;

Serial.print("Pressure (hPa): ");

Serial.println(pressure);

//uncomment if you want to get temperature in Fahrenheit

/*temperatureF = 1.8 * bme.readTemperature() + 32;

Serial.print("Temperature (ºC): ");

Serial.println(temperatureF);*/

// set the fields with the values

ThingSpeak.setField(1, temperatureC);

//ThingSpeak.setField(1, temperatureF);

ThingSpeak.setField(2, humidity);

ThingSpeak.setField(3, pressure);

// Write to ThingSpeak. There are up to 8 fields in a channel, allowing you to store up to 8 different

// pieces of information in a channel.  Here, we write to field 1.

int x = ThingSpeak.writeFields(myChannelNumber, myWriteAPIKey);

if(x == 200){

Serial.println("Channel update successful.");

}

else{

Serial.println("Problem updating channel. HTTP error code " + String(x));

}

lastTime = millis();

}

}

void initBME(){

if (!bme.begin(0x77)) {

Serial.println("Could not find a valid BME280 sensor, check wiring!");

while (1);

}

}

void setup() {

Serial.begin(115200);  //Initialize serial

initBME();

//WiFi.mode(WIFI_STA);

ThingSpeak.begin(client);  // Initialize ThingSpeak

}