/*

Rui Santos

Complete project details at https://RandomNerdTutorials.com/esp-mesh-esp32-esp8266-painlessmesh/

Permission is hereby granted, free of charge, to any person obtaining a copy

of this software and associated documentation files.

The above copyright notice and this permission notice shall be included in all

copies or substantial portions of the Software.

*/

#include <Adafruit_Sensor.h>

#include <Adafruit_BME280.h>

#include "painlessMesh.h"

#include <Arduino_JSON.h>

// MESH Details

#define   MESH_PREFIX     "AEMESH" //name for your MESH

#define   MESH_PASSWORD   "MESHpassword" //password for your MESH

#define   MESH_PORT       5555 //default port

//BME object on the default I2C pins

//Adafruit_BME280 bme;

//Number for this node

int nodeNumber = 18;

//String to send to other nodes with sensor readings

String readings;

Scheduler userScheduler; // to control your personal task

painlessMesh  mesh;

// User stub

void sendMessage() ; // Prototype so PlatformIO doesn't complain

String getReadings(); // Prototype for sending sensor readings

//Create tasks: to send messages and get readings;

Task taskSendMessage(TASK_SECOND * 5 , TASK_FOREVER, &sendMessage);

// String getReadings ()

// JSONVar jsonReadings;

// jsonReadings["node"] = nodeNumber;

// jsonReadings["temp"] = bme.readTemperature();

// jsonReadings["hum"] = bme.readHumidity();

// jsonReadings["pres"] = bme.readPressure()/100.0F;

// readings = JSON.stringify(jsonReadings);

// return readings;

// }

void sendMessage () {

String msg = "";

mesh.sendBroadcast(msg);

}

//Init BME280

// void initBME(){

// if (!bme.begin(0x77)) {

//   Serial.println("Could not find a valid BME280 sensor, check wiring!");

//   while (1);

// }

// }

// Needed for painless library

void receivedCallback( uint32_t from, String &msg ) {

Serial.printf("Received from %u msg=%s\n", from, msg.c_str());

JSONVar myObject = JSON.parse(msg.c_str());

int node = myObject["node"];

double temp = myObject["temp"];

double hum = myObject["hum"];

double pres = myObject["pres"];

Serial.print("Node: ");

Serial.println(node);

Serial.print("Temperature: ");

Serial.print(temp);

Serial.println(" C");

Serial.print("Humidity: ");

Serial.print(hum);

Serial.println(" %");

Serial.print("Pressure: ");

Serial.print(pres);

Serial.println(" hpa");

}

void newConnectionCallback(uint32_t nodeId) {

Serial.printf("New Connection, nodeId = %u\n", nodeId);

}

void changedConnectionCallback() {

Serial.printf("Changed connections\n");

}

void nodeTimeAdjustedCallback(int32_t offset) {

Serial.printf("Adjusted time %u. Offset = %d\n", mesh.getNodeTime(),offset);

}

void setup() {

Serial.begin(115200);

//  initBME();

//mesh.setDebugMsgTypes( ERROR | MESH_STATUS | CONNECTION | SYNC | COMMUNICATION | GENERAL | MSG_TYPES | REMOTE ); // all types on

mesh.setDebugMsgTypes( ERROR | STARTUP );  // set before init() so that you can see startup messages

mesh.init( MESH_PREFIX, MESH_PASSWORD, &userScheduler, MESH_PORT );

mesh.onReceive(&receivedCallback);

mesh.onNewConnection(&newConnectionCallback);

mesh.onChangedConnections(&changedConnectionCallback);

mesh.onNodeTimeAdjusted(&nodeTimeAdjustedCallback);

userScheduler.addTask(taskSendMessage);

taskSendMessage.enable();

}

void loop() {

// it will run the user scheduler as well

mesh.update();

}

/*********

Rui Santos

Complete project details at https://randomnerdtutorials.com/esp32-web-server-with-bme280-mini-weather-station/

*********/

// Load Wi-Fi library

#include <WiFi.h>

#include <Wire.h>

#include <Adafruit_BME280.h>

#include <Adafruit_Sensor.h>

#define SEALEVELPRESSURE_HPA (1013.25)

Adafruit_BME280 bme;

// Replace with your network credentials

const char* ssid     = "US_Guest";

const char* password = "";

// Set web server port number to 80

WiFiServer server(80);

// Variable to store the HTTP request

String header;

// Current time

unsigned long currentTime = millis();

// Previous time

unsigned long previousTime = 0;

// Define timeout time in milliseconds (example: 2000ms = 2s)

const long timeoutTime = 2000;

void setup() {

Serial.begin(115200);

bool status;

// default settings

// (you can also pass in a Wire library object like &Wire2)

//status = bme.begin();

if (!bme.begin(0x77)) {

Serial.println("Could not find a valid BME280 sensor, check wiring!");

while (1);

}

// Connect to Wi-Fi network with SSID and password

Serial.print("Connecting to ");

Serial.println(ssid);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

// Print local IP address and start web server

Serial.println("");

Serial.println("WiFi connected.");

Serial.println("IP address: ");

Serial.println(WiFi.localIP());

server.begin();

}

void loop(){

WiFiClient client = server.available();   // Listen for incoming clients

if (client) {                             // If a new client connects,

currentTime = millis();

previousTime = currentTime;

Serial.println("New Client.");          // print a message out in the serial port

String currentLine = "";                // make a String to hold incoming data from the client

while (client.connected() && currentTime - previousTime <= timeoutTime) {  // loop while the client's connected

currentTime = millis();

if (client.available()) {             // if there's bytes to read from the client,

char c = client.read();             // read a byte, then

Serial.write(c);                    // print it out the serial monitor

header += c;

if (c == '\n') {                    // if the byte is a newline character

// if the current line is blank, you got two newline characters in a row.

// that's the end of the client HTTP request, so send a response:

if (currentLine.length() == 0) {

// HTTP headers always start with a response code (e.g. HTTP/1.1 200 OK)

// and a content-type so the client knows what's coming, then a blank line:

client.println("HTTP/1.1 200 OK");

client.println("Content-type:text/html");

client.println("Connection: close");

client.println();

// Display the HTML web page

client.println("<!DOCTYPE html><html>");

client.println("<head><meta name=\"viewport\" content=\"width=device-width, initial-scale=1\">");

client.println("<link rel=\"icon\" href=\"data:,\">");

// CSS to style the table

client.println("<style>body { text-align: center; font-family: \"Trebuchet MS\", Arial;}");

client.println("table { border-collapse: collapse; width:35%; margin-left:auto; margin-right:auto; }");

client.println("th { padding: 12px; background-color: #0043af; color: white; }");

client.println("tr { border: 1px solid #ddd; padding: 12px; }");

client.println("tr:hover { background-color: #bcbcbc; }");

client.println("td { border: none; padding: 12px; }");

client.println(".sensor { color:white; font-weight: bold; background-color: #bcbcbc; padding: 1px; }");

// Web Page Heading

client.println("</style></head><body><h1>ESP32 BME280 Readings</h1>");

client.println("<table><tr><th>MEASUREMENT</th><th>VALUE</th></tr>");

client.println("<tr><td>Temp. Celsius</td><td><span class=\"sensor\">");

client.println(bme.readTemperature());

client.println(" *C</span></td></tr>");

client.println("<tr><td>Temp. Fahrenheit</td><td><span class=\"sensor\">");

client.println(1.8 * bme.readTemperature() + 32);

client.println(" *F</span></td></tr>");

client.println("<tr><td>Pressure</td><td><span class=\"sensor\">");

client.println(bme.readPressure() / 100.0F);

client.println(" hPa</span></td></tr>");

client.println("<tr><td>Approx. Altitude</td><td><span class=\"sensor\">");

client.println(bme.readAltitude(SEALEVELPRESSURE_HPA));

client.println(" m</span></td></tr>");

client.println("<tr><td>Humidity</td><td><span class=\"sensor\">");

client.println(bme.readHumidity());

client.println(" %</span></td></tr>");

client.println("</body></html>");

// The HTTP response ends with another blank line

client.println();

// Break out of the while loop

break;

} else { // if you got a newline, then clear currentLine

currentLine = "";

}

} else if (c != '\r') {  // if you got anything else but a carriage return character,

currentLine += c;      // add it to the end of the currentLine

}

}

}

// Clear the header variable

header = "";

// Close the connection

client.stop();

Serial.println("Client disconnected.");

Serial.println("");

}

}

Ultimate Code:

*/

#include <Adafruit_Sensor.h>

#include <Adafruit_BME280.h>

#include "painlessMesh.h"

#include <Arduino_JSON.h>

// Load Wi-Fi library

#include <WiFi.h>

#include <Wire.h>

#include <Adafruit_BME280.h>

#include <Adafruit_Sensor.h>